|  |  | Memo |  |  |
| --- | --- | --- | --- | --- |
| To: | FEMA Technical Reviewer |  |  |  |
| From: | HDR Engineering, Inc. 3200 E. Camelback Rd, Suite 350 Phoenix, AZ 85018 |  | Project: | Verde River Flood Hazard Assessment |
| CC: | File & Charlie Cave |  |  |  |
| Date: | December 2010 |  | Job No: | 114474 |

RE:  4 Percent Annual Chance Equivalent Discharge

In support of the Verde River Flood Hazard Assessment Technical Data Notebook deliverable and the currently effective FEMA RISK Map standards, the 4 Percent Annual Chance Equivalent Discharges (25-Year Storm Events) have been prepared as an addendum to the currently prepared Hydrologic Analysis report to support FEMA review.

The hydrologic discharges were prepared using HEC-SSP (Version 1.1), based on frequency in percent (a user specified frequency ordinate of 4 percent) within the frequency ordinate definition. All values for skew, probability, plotting, confidence limits and historic period data were maintained as specified with the approved final Hydrologic Analysis report or HEC-SSP modeling dated April 2010.

As a result, the annual chance of occurrence for Gage 9504000 and Gage 9506000 identifies computed discharges of 38,980 cfs and 66,200 cfs respectively. (Refer to attached report summary printouts from HEC-SSP).

With respect to the discharges from Gage 9504000 and Gage 9506000, interpolated discharges based on drainage area (in square miles) were prepared for locations of major confluence along the Verde River.  These locations are 89A Bridge, Oak Creek, Beaver Creek and West Clear Creek that are located between the Gages evaluated here-in.  Tabular comparison to the 10- , 2-, 1- and 0.2 – percent annual chance of occurrence discharges can be viewed in the attached computational table.

In support of the Hydrologic Analysis and the TDN the 4 Percent Annual Chance Equivalent Discharges are provided below.

| Location | 4-percent annual chance flow * (cfs) | 4-percent annual chance flow |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Drainage Area |  |  |  |  |
|  |  | (cfs/mi2) |  |  |  |  |
| 9504000 | 38,980 | 12 |  |  |  |  |
| 89A Bridge | 40,500 | 12 |  |  |  |  |
| Oak Creek | 47,100 | 12 |  |  |  |  |
| Beaver Creek | 53,500 | 12 |  |  |  |  |
| Clear Creek | 57,600 | 12 |  |  |  |  |
| 9506000 | 66,200 | 14 |  |  |  |  |